# MULTIMEDIA TRAINING KIT

Exercises: Antenna

Developed by: Onno W. Purbo

### Exercise #1

Imagine a typical situation:

- You build your own antenna else own a new antenna.

- You want to know the gain of the antenna.

What you will need:

- PC with wireless LAN card, or notebook with WiFi enabled / PCMCIA Wireless card.

- Download Netstumbler from http://www.netstumbler.com

- Access Point with external antenna connector.

- The unknown gain external antenna.

- Pigtail with known loss, normally about 0.25 dB losses for short 0.5 meters pigtail.

Steps:

Prepare the Access Point for reference antenna measurement.

- Plug the original Access Point omni directional antenna. It is normally a 3 dBi antenna.

- If the Access Point has two antennas, set to use only one of them.

- Set the SSID, Channel.

- Install the Access Point and your PC / notebook > 50 meter apart.

- Let the Access Point running.

Measure the transmission of reference antenna.

- Use your notebook / PC with netstumbler to measure the signal strength of the Access Point at the distance.

- Please write down the received signal strength.

Measure the unknown gain antenna

- Replace the reference omni directional antenna.

- Use the unknown gain antenna as the replacement; connect it via pigtail if needed.

- Point the antenna to the notebook which netstumbler running.

- Write down the received signal using the unknown gain antenna.

Do a simple calculation to get the antenna gain

Antenna Gain (dBi) = Received signal (unknown antenna) + Pigtail Loss (0.25 dB)

  - Received signal (omni antenna) + Gain Omni Antenna (3dBi).

### Exercise #2

Imagine a typical situation:

- You build your own antenna else own a new antenna.

- You want to know the Front To Back Ratio (F/B) of the antenna.

What you will need:

- PC with wireless LAN card, or notebook with WiFi enabled / PCMCIA Wireless card.

- Download Netstumbler from http://www.netstumbler.com

- Access Point with external antenna connector.

- The external antenna.

- Pigtail with known loss, normally about 0.25 dB losses for short 0.5 meters pigtail.

Steps:

Prepare the Access Point

- If the Access Point has two antennas, set to use only one of them.

- Set the SSID, Channel.

- Install the Access Point and your PC / notebook > 50 meter apart.

- Let the Access Point running.

Measure the front signal from antenna.

- Plug the antenna to the access point.

- Pointing the antenna towards the notebook.

- Use your notebook / PC with netstumbler to measure the signal strength of the Access Point at the distance.

- Please write down the received signal strength.

Measure the back signal of the antenna

- Rotate the antenna 180 degree.

- Write down the received signal using the unknown gain antenna.

Do a simple calculation to get F/B of the antenna

F/B (dB) = Received front signal – Received back signal.

Note

- You can measure your antenna radiation pattern & side lobes by slowly rotating the antenna & measure the signal.