# Noise Source and Solution

Reference:

Jack Unger, Deploying License-Free Wireless Wide-Area Networks: Best practices for planning and deployment of broadband WWANs, Cisco Press, 2003

| Souce of Noise | Symptom | Solution (Before Beginning Wireless Service) | Solution (After Beginning Wireless Service) |
| --- | --- | --- | --- |
| Antenna directional patterns ignored. (Antenna pattern is too broad). | Reduced throughput caused by picking up noise from unwanted source. | Choose a more directional antenna for your Access Point (AP) with a coverage pattern that covers only the desired area. | Research, design, purchase, install, and test a new, more directional antenna system. |
| High noise level from other transmitters near your AP location. These transmitters might be out-of-band (AM, FM, TV broadcasting, paging, or two-way radio) or in-band (other networks in the same license-free band). | Reduced throughput caused by picking up noise from nearby transmitters. | Perform an RF site survey using a spectrum analyzer. | For in-band noise: Temporarily shut down all the transmitters on your network and use a spectrum analyzer to attempt to locate the source of interference. If the noise cannot be reduced, it might be necessary for you to move your AP to a new location. If you locate the interference source, reconfigure your antenna system polarization, directivity, or downtilt to reduce the noise. For out-of-band noise: Temporarily add a bandpass filter between the antenna and the radio to see if the throutput improves. |
| Interference from another network that is on the same DSSS frequency channel. | Reduced throughput | Choose a DSSS channel with as little use as possible, as determined by a spectrum analyzer. | Change DSSS channels; choose one with the least amount of use, as determined by a spectrum analyzer. |
| Interference from other networks using the same FHSS hoping sequence or using another hopping set. | Reduced throughput | Coordinate your choice of hopping set and hopping sequence. Use the same hopping set as the other FHSS networks in your area but a different hopping sequence with that set. | Change your choice of hopping set and hopping sequence. Use the same hopping set as the other FHSS networks in your area but a different hopping sequence within that set. |
| Interference from another radio or from another AP in your network. | Reduced throughput on both your Aps. | Prepare a frequency plan. Design your network to ensure adequate isolation between your antenna systems that operate on the same frequency. Use a combination of antenna directivity, cross polarization, cross polarization, antenna downtilting, vertical separation, horizontal separation, and obstruction isolation between your antennas. | Temporary shutdown one of your networks while you add isolation between your antenna systems by using a combination of the techniques listed at the left. |
| Wireless packet collisions between end users on your own network. | Reduced throughput under moderate and heavy traffic levels. | Select equipment that is capable of polling end users rather than using the standard carrier sense multiple access with collision avoidance (CSMA/CA) protocol that allows hidden-node end users to collide with each other. | Upgrade both AP and end user software to a version that provides end user polling. If this is not possible, install a bandwidth manager that can throttle (limit) each end user to predetermined bandwidth level. If necessary, throttle end user upload speeds at a level less than their download speeds. |
| Multipath | High error rates with reduced throughput. | Avoid placing wireless Aps in reflection-prone locations. Use circular polarized antenna whenever possible if you must operate in an area with reflective building, standing bodies of water, or a large sections of concrete parking lots. | Move your Aps to less reflective areas; temporarily substitute a circularly polarized antenna system on your AP and test to see if the throughput under load is improved. |