IEEE 802.20-PD-08

Date: September 6, 2005, approved by WG at July, 2005 Plenary

# 802.20 Channel Models Document for IEEE 802.20 MBWA System Simulations – 802.20-PD-08

This document is a Permanent Document of IEEE Working Group 802.20. Permanent Documents (PD) are used in facilitating the work of the WG and contain information that provides guidance for the development of  802.20 standards.

Contents

1	Introduction	1

1.1	Purpose	1

1.2	Scope	1

1.3	Abbreviations	1

2	SISO Channel Models	1

2.1	Link Level Simulation	2

2.2	System Level Simulation	3

3	MIMO Channel Models	3

3.1	Introduction	3

3.2	Spatial Channel Characteristics	3

3.3	MIMO Channel Model Classification	4

3.4	MBWA Channel Environments	4

3.5	General Description	5

3.6	MIMO Correlation Channel Matrices	8

3.6.1	Definition of Correlation Channel Matrices	9

3.6.2	Generation of a MIMO Channel Using Correlation Matrix Approach	9

3.7	Link Level Spatial Channel Model Parameter Summary	10

4	MIMO Channel Model for System Level Simulations	12

4.1	Introduction	12

5	ASSUMPTIONS AND PARAMETERS	13

5.1	Antenna Topologies	13

5.2	Spatial Parameters for the Base Station	16

5.2.1	BS Angles of Departure and Arrival	16

5.2.2	BS Angle Spread	16

5.2.3	BS Power Azimuth Spectrum	16

5.3	Spatial Parameters for the Mobile Station	17

5.3.1	MS Antenna Topologies	17

5.3.2	MS Angle Spread	17

5.3.3	MS Angle of Arrival	17

5.3.4	MS Power Azimuth Spectrum	18

5.3.5	MS Direction of Travel	18

5.3.6	Doppler Spectrum	19

5.4	Definitions, Parameters, and Assumptions	19

5.5	MIMO Channel Environments	21

6	Procedure for Generating SCM Parameters	23

6.1	Introduction	23

6.1.1	Generating Model Parameters for Urban and Suburban Macrocell Environments	23

6.1.2	Generating Model Parameters for Urban Microcell Environments	24

6.2	Generating SCM Coefficients	24

7	A Method of Generating Spatial Correlation Coefficients for MIMO Channels	26

8	References and Bibliography	28

# Introduction

This document describes the SISO and MIMO radio channel models that are to be used for simulating proposals for the future 802.20 standard.

## Purpose

This document specifies channel models for simulations of MBWA Air Interface schemes at link level, as well as system level.

## Scope

The scope of this document is to define the specifications of mobile broadband wireless channel models.

## Abbreviations

AoA		Angle of Arrival

AoD		Angle of Departure

AS		AngularSpread

BS		Base Station

DoT		Direction of Travel

DS		Delay Spread

MEA		Multi-Element Array

MIMO	Multiple-Input Multiple Output

MISO		Multiple-Input Single-Output

MS		Mobile Station

PAS		Power Azimuth Spectrum

PDP		Power Delay Profile

PL		Path Loss

Rx		Receiver

SCM		Spatial Channel Model

SISO		Single-Input Single Output

SIMO		Single-Input Multiple-Output

TE		Test Environment

Tx		Transmitter

ULA		Uniform Linear Array

# SISO Channel Models

SISO systems shall use the ITU model in simulations.

## Link Level Simulation

| Models |  | case-i |  | case-ii |  | case-iii |  | case-Iv |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDP |  | Pedestrian-A |  | Vehicular-A |  | Pedestrian-B (Phase I) |  | Vehicular-B (Phase I) |  |
| Number of Paths |  | 4 |  | 6 |  | 6 |  | 6 |  |
| Relative Path power (dB) | Delay (ns) | 0 | 0 | 0 | 0 | 0 | 0 | -2.5 | 0 |
|  |  | -9.7 | 110 | -1.0 | 310 | -0.9 | 200 | 0 | 300 |
|  |  | -19.2 | 190 | -9.0 | 710 | -4.9 | 800 | -12.8 | 8900 |
|  |  | -22.8 | 410 | -10.0 | 1090 | -8.0 | 1200 | -10.0 | 12900 |
|  |  |  |  | -15.0 | 1730 | -7.8 | 2300 | -25.2 | 17100 |
|  |  |  |  | -20.0 | 2510 | -23.9 | 3700 | -16.0 | 20000 |
| Speed (km/h) |  | 3, 30, 120 |  | 30, 120, 250 |  | 3 [EditorNote: Consistency check with EV document when approved ] |  | 30, 120, 250 [Ed.Note: Consistency check with EV doc when approved] |  |

Table 2.1-1 Summary of SISO Channel Model Parameters

## System Level Simulation

| Channel Scenario | Suburban Macro (Phase I) | Urban Macro | Urban Micro |  |
| --- | --- | --- | --- | --- |
| Lognormal shadowing standard deviation | 10dB | 10dB | NLOS: 10dB LOS: 4dB |  |
| Pathloss model (dB), d is in meters | 31.5 + 35log10(d) | 34.5 + 35log10(d) | NLOS:34.53+38log10(d) LOS:30.18 + 26*log10(d) |  |

Table 2.2-1 SISO Channel Environment Parameters

Please see Section 3.5 for example of how the correlation matrix approach to MIMO channel models collapses to the ITU-R model for SISO systems.

For calculations using a center frequency as a variable, for example path loss, 1900 MHz is assumed.

See Section 3.4 for Definitions

# MIMO Channel Models

## Introduction

In this Chapter, a set of spatial channel model parameters are specified that have been developed to characterize the particular features of MIMO radio channels. SISO channel models provide information on the distributions of signal power level and Doppler shifts of received signals. MIMO channel models, which are based on the classical understanding of multi-path fading and Doppler spread, incorporate additional concepts such as Angular Spread, Angle of Arrival, Power-Azimuth-Spectrum (PAS), and the antenna array correlation matrices for the transmitter (Tx) and receiver (Rx) combinations.

## Spatial Channel Characteristics

Mobile broadband radio channel is a challenging environment, in which the high mobility causes rapid variations across the time-dimension, multi-path delay spread causes severe frequency-selective fading, and angular spread causes significant variations in the spatial channel responses. For best performance, the Rx & Tx algorithms must accurately track all dimensions of the channel responses (space, time, and frequency). Therefore, a MIMO channel model must capture all the essential channel characteristics, including

- Spatial characteristics (Angle Spread, Power Azimuth Spectrum, Spatial correlations),

- Temporal characteristics (Power Delay Profile),

- Frequency-domain characteristics (Doppler spectrum).

In MIMO systems, the spatial (or angular) distribution of the multi-path components is important in determining system performance. System capacity can be significantly increased by exploiting rich multi-path scattering environments.

## MIMO Channel Model Classification

There are three main approaches to MIMO channel modeling: the correlation model, the ray-tracing model, and the scattering model.  The properties of these models are briefly described as follows:

- Correlation Model: This model characterizes spatial correlation by a linear combiniation of independent complex channel matrices at the transmitter and receiver. For multipath fading channels, the ITU (SISO) model [25] is used to generate the power delay profile and Doppler spectrum.  Since this model is based on ITU generalized tap delay line channel model, the model is simple to use and backward compatible with existing ITU channel profiles.

- Ray-Tracing Model: In this approach, exact locations of the primary scatterers, their physical characteristics, as well as the exact location of the transmitter and receiver are assumed known. The resulting channel characteristics are then predicted by summing the contributions from a large number of the propagation paths from each transmit antenna to each receive antenna. This technique provides fairly accurate channel prediction by using site-specific information, such as database of terrain and buildings. For modeling outdoor environments this approach requires detailed terrain and building databases.

- Scattering Model: This model assumes a particular statistical distribution of scatterers.  Using this distribution, channel models are generated through simulated interaction of scatterers and planar wave-fronts.  This model requires a large number of parameters.

## MBWA Channel Environments

The following channel environments shall be considered for MBWA system simulations:

- Suburban macro-cell

  - Large cell radius (approximately 1-6 km BS to BS distance)

  - High BS antenna positions ( above  rooftop heights, between 10-80m (typically 32m))

  - Moderate to high delay  spreads and low angle spreads

  - High range of mobility (0 – 250 km/h)

- Urban macro-cell

  - Large cell radius (approximately 1-6 km BS to BS distance)

  - High BS antenna positions ( above  rooftop heights, between 10-80m (typically 32m))

  - Moderate to high delay and angle spread

  - High range of mobility (0 – 250 km/h)

- Urban micro-cell

  - Small cell radius (approximately 0.3 – 0.5 km BS to BS distance)

  - BS antenna positions (at  rooftop heights or lower (typically 12.5m))

  - High angle spread and moderate delay spread

  - Medium range of mobility (0 – 120 km/h)

  - The model is sensitive to antenna height and scattering environment (such as street layout, LOS)

## General Description

In this section we will describe a MIMO channel model that captures the above characteristics and that can be collapsed to an underlying SISO ITU channel model. We will first start by describing a basic underlying model for a SISO link and then generalize this model in an incremental fashion to describe MIMO channels. A simple model for a SISO link is given by

where is the complex tap gain which is assumed to be a complex Gaussian random variable with zero mean and variance , is the corresponding delay, and  is the number of taps in the channel profile.  Note that in this model we assume that the time delays  changes very slowly with time such that we can assume that they are constant. Also, the tap gains are time varying in general.  The tap gains will have an autocorrelation function that will depend on the scattering process as well as the mobility of the transmitter and/or the receiver. In the case of uniform scattering, this will be the classical Jakes spectrum model with   where  is the maximum delay spread of the channel. Note that we also that complex tap gains are independent.

Now in order to generate an equivalent digital channel model, the transmit and receive filtering of the channel needs to be taken into consideration as follows. Let  be the transmit pulse shape and be the receive filter impulse response.  Let us also assume that the transmitted signal from some transmitter in the network is

where  is the digital symbol being transmitted and  is the symbol period. The corresponding received signal is

where

is the overall transmitted signal that includes the effects of pulse shaping, transmit filtering, and received filtering and .  Hence, the received signal can be rewritten as

Note that in the above equation, we assumed that the tap gains  will remain constant over a symbol period. Let us now assume that the received signal is over sampled by a factor of at the receiver, i.e. the sampling instants are

Then we will have

Let . Also, a reasonable assumption to make here is that the overall channel response (including pulse shaping and transmit and receive filtering) will have a finite duration. Hence, we will have

where

is the equivalent digital channel tap.   Let us consider the symbol rate equivalent channel (i.e., the case when the output of the receive filter is sampled at the symbol rate):

or  in a vector form

The model in () and () describes a SISO ISI channel. The time correlation behavior of the channel is reflected in the time correlation behavior (which depends on the Doppler spread of the channel) of the tap gains’s. The frequency correlation behavior of the channel is reflected in the pulse shaping matrix and its dependency on the time delays’s (and hence its dependency on the channel delay spread).

A MIMO channel with transmit and receive antennas is made up of  SISO links with multipth components. The channel coefficients for one of  multi-path components are given by a complex matrix. We denote the channel matrix for the i-th multi-path component as, where . The broadband MIMO radio channel transfer matrix can be modeled as

where  and

A key and a very reasonable assumption used in the model in ()  is that all SISO links will have the same underlying multipath structure, i.e. they will have the same delay spread and the same multipath delays  ’s. The reason for this is that the path delays and the delay spread are generally dictated by the large scattering and reflecting structure in the propagation environment which will common to all transmitting and receiving antennas (See [26] and [27] for details). However, the individual gains taps across the different SISO links are different, in general. The relationship between these channel taps will, in general, depend on the geometry of the transmitting and receiving antenna arrays, the angle of arrival/departure  and angle spread  of each multipath component (Again, see [26] and [27] for details).  Let  be the -th column of the matrix. Similarly, let  be the -th raw .  Clearly,  represents the -th multipath component tap gains from the -th transmit antenna to the  receive antennas and  represents  -th multipath component tap gains to the -th receive antenna from the  transmit antennas.  The spatial correlation of the transmitting and receiving arrays, i.e. the correlation of  the  across the transmitting and/or the receiving arrays  as  a function of the arrays geometry and the multipath components angle of arrival/departure and their corresponding angle spreads, is captured in the transmit and receive correlation matrices, defined as:

Note that these correlation matrices depend on  to the extent that each multiptah component may have a different angle of arrival/departure and the corresponding angle spread. In addition, and as we stated above, these correlation matrices also depend on the respective array geometry the radiation pattern of each antennas element.

Figure 1  A MIMO channel with N transmit and M receive antennas

## MIMO Correlation Channel Matrices

Non-SISO systems shall use a correlation matrix approach in simulations. The correlation matrices are only antenna system dependent. The correlation matrices may be generated by using the SCM approach (SECTION 6) or computed analytically based on the PAS distribution and array geometry (SECTION 7). The matrices used shall be submitted as part of the simulation report.  There are two matrices that need to be considered, the transmit correlation matrix (TCM) and the receive correlation matrix (RCM).

### Definition of Correlation Channel Matrices

In the correlation matrix approach, the channel from any of the  transmit antennas to the  receive antenna elements is generated from  independent channels from that transmit antenna to the receive antennas. That is, for any given channel tap, we will have

Where is the channel vector from the -th transmitting antenna to the  receive antennas, is the underlying  independent Gaussian channel vector  (i.e. it is the channel vector from the -th transmitting antenna to the  receive antennas if the receive antennas were uncorrelated), and is the square root of the channel receive correlation matrix. Please note that the dimensions of , , and  are , , and  , respectively.  In addition, we note that each ITU channel profile defines a number of taps with a corresponding tap delay and average tap power.  The above description for the channel vector  is repeated for each channel tap. Moreover, please note that the underlying independent Gaussian channel vector  is completely different (i.e. independent) for each tap. Note that, when there is only one transmit antenna and one receive antenna,  is simply 1 and the above reduces to the scalar ITU channel model.

In a similar fashion, let us now consider the channel from the  transmit antennas to any of the  receive antennas.  The channel row-vector corresponding to the channels from all the  transmit antennas to the -th receive antenna is related to the underlying independent Gaussian channel row-vector (i.e the channel row-vector from all -transmit antennas to the -th receive antenna if the transmit antennas were uncorrelated) by

where   is the square root of the transmit array correlation matrix. We note that the dimensions of , , and are , , and   respectively .

### Generation of a MIMO Channel Using Correlation Matrix Approach

Some of the parameters that can be used in the correlation channel model are shown in Table 3.5.2-1. In order to generate a MIMO channel, we first need to have a pair of transmit and receive  and  correlation matrices. These are generated for each mobile station (MS) and base station (BS) based on the number of antennas, antenna spacing, number of clusters, power azimuth spectrum (PAS), azimuth spread (AS), and angle of arrival (AoA). In addition to the correlation matrices, we also need to specify an underlying ITU SISO model from Table 3.5.2-1 and choose the mobile speed.

As an example, a MIMO link with transmit and receive antennas can be then generated as follows:

- Generate  SISO links based on the chosen ITU profiles as follows

  - Let  and  represent the power-delay profile for the specified ITU channel model

  - Generate independent Rayleigh fading processes each having a Doppler spread  (function of the chosen mobile speed). The number of samples in each of the fading processes is given by the required number of symbols at the specified sampling rate (See Section 3.5 above).

  - Scale the -th Rayleigh process by  where

  - Generate the pulse shaping matrix  as defined in equation () above.

  - Compute the channels taps; processes according to (). This will result in Rayleigh fading processes where  is the number of taps in the digital channel corresponding to the specified ITU channel model.

- Given the SISO links  generated in step 1 above, each is described by  processes, we define the following  -th tap gain matrix for every channel sample (i.e. for every )

3. Color the tap gain matrix by the receive and transmit correlation matrices as follows

## Link Level Spatial Channel Model Parameter Summary

This section describes link-level channel modeling parameters.

The following table summarizes the physical parameters to be used for link level modeling.

| Models |  |  | case-i |  | case-ii |  | case-iii |  | case-Iv |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDP |  |  | Pedestrian-A |  | Vehicular-A |  | Pedestrian-B (Phase I) |  | Vehicular-B (Phase I) |  |  |
| Number of Paths |  |  | 4 |  | 6 |  | 6 |  | 6 |  |  |
| Relative Path power (dB) |  | Delay (ns) | 0 | 0 | 0 | 0 | 0 | 0 | -2.5 | 0 |  |
|  |  |  | -9.7 | 110 | -1.0 | 310 | -0.9 | 200 | 0 | 300 |  |
|  |  |  | -19.2 | 190 | -9.0 | 710 | -4.9 | 800 | -12.8 | 8900 |  |
|  |  |  | -22.8 | 410 | -10.0 | 1090 | -8.0 | 1200 | -10.0 | 12900 |  |
|  |  |  |  |  | -15.0 | 1730 | -7.8 | 2300 | -25.2 | 17100 |  |
|  |  |  |  |  | -20.0 | 2510 | -23.9 | 3700 | -16.0 | 20000 |  |
| Speed (km/h) |  |  | 3, 30, 120 |  | 30, 120, 250 |  | 3, [Ed.Note: Subject to consistency with EV doc when approved ] |  | 30, 120, 250 [Ed. Note: Subject to consistency with EV doc when approved ] |  |  |
| Mobile Station | Topology |  | 0.5λ |  | 0.5λ |  | 0.5λ |  | 0.5λ |  |  |
|  | PAS |  | - LOS on: Fixed AoA for LOS component, remaining power has 360 degree uniform PAS. 2) LOS off: PAS with a Laplacian distribution, RMS angle spread of 35 degrees per path |  | RMS angle spread of 35 degrees per path with a Laplacian distribution Or 360 degree uniform PAS |  | RMS angle spread of 35 degrees per path with a Laplacian distribution |  | RMS angle spread of 35 degrees per path with a Laplacian distribution Or 360 degree uniform PAS |  |  |
|  | DoT (degrees) |  | 0 |  | 22.5 |  | -22.5 |  | 22.5 |  |  |
|  | AoA (degrees) |  | 22.5 (LOS component) 67.5 (all other paths) |  | 67.5 (all paths) |  | 67.5 (all paths) |  | 67.5 (all paths) |  |  |
| Base Station | Topology |  | Reference: ULA with 0.5λ-spacing or 4λ-spacing or 10λ-spacing |  |  |  |  |  |  |  |  |
|  | PAS |  | Laplacian distribution with RMS angle spread of 2 degrees or 5 degrees, per path depending on AoA/AoD |  |  |  |  |  |  |  |  |
|  | AoD/AoA (degrees) |  | 50 for 2 RMS angle spread per path 20 for 5 RMS angle spread per path |  |  |  |  |  |  |  |  |

Table 3.7.1-1 Summary of Link Level Channel Model Parameters

# MIMO Channel Model for System Level Simulations

## Introduction

The spatial channel model for MBWA system-level simulations is described in this chapter. As in the link level simulations, the description is in the context of a downlink system where BS transmits to a MS; however the methodology described here can be applied to the uplink as well. The goal of this chapter is to define the methodology and parameters for generating the spatial and temporal MIMO channel model coefficients for MBWA system simulations.

As opposed to link level simulations where only considering the case of a single BS transmitting to a single MS, the system level simulations typically consist of multiple cells/sectors, BSs, and MSs. Performance metrics such as data throughputs are collected over D drops, where a "drop" is defined as a simulation run for a given number of cells/sectors, BSs, and MSs, over a specified number of frames.

During a drop, the channel undergoes fading according to the speed of MSs. Channel state information is fed back from the MSs to the BSs, and the BSs use schedulers to determine which user(s) to transmit to. Typically, over a series of drops, the cell layout is fixed, but the locations of the MSs are still random variables at the beginning of each drop.

The overall procedure for generating the channel matrices consists of three basic steps:

- Specify an environment, i.e., suburban macro, urban macro, or urban micro.

- Obtain the parameters to be used in simulations, associated with that environment.

- Generate the channel coefficients based on the parameters.

The following sections describe the details of overall procedure. The figure below provides a flow chart for generating channel coefficients.

Figure 4-1  The flow chart for the generation of MIMO channel model coefficients

# ASSUMPTIONS AND PARAMETERS

## Antenna Topologies

At the BS, a linear antenna array is assumed. Three values for reference antenna element spacing are defined as: 0.5, 4, and 10.

The 3-sector antenna pattern is used for BS, which is plotted in Figure 5.1.2 and is specified by

is defined as the angle between the direction of interest and the broadside of the antenna array.  is the 3dB beam-width in degrees, and  is the maximum attenuation. For a 3 sector scenario  is 70 degrees, and = 20dB. The term broadside refers to the direction from which the signal is coming perpendicularly to the MEA.  An antenna array shows the maximum gain at its broadside direction. The antenna broadside pointing direction is illustrated by Figure 2.2 for a 3-sector scenario.  The antenna gain, as specified by 3GPP/3GPP2 document [13] which assumes the frequency band range is 2-3 GHz, is 14 dBi for a 3-sector scenario.

Figure 5.1.2-1  Antenna pattern for 3-sector cells

Figure 5.1.1-2 Illustration of the broadside pointing direction of antenna array for 3-sector cells

For a 6 sector scenario,  is 35 degree,  =23dB, which results in the antenna pattern shown in the Figure 2.3 below, and the broadside pointing direction illustrated by Figure 2.4. The gain specified by 3GPP document [13] is 17dBi for a 6 sector scenario.

Figure 5.1.1-3 Antenna Pattern for 6-sector cells

Figure 5.1.1-4 Broadside Pointing Direction for 6-sector cells

## Spatial Parameters for the Base Station

### BS Angles of Departure and Arrival

The Angle of Departure (AoD) is defined to be the mean angle with which a departing ray’s power is transmitted by the BS array with respect to the broadside. The Angle of Arrival (AoA) is defined to be the mean angle with which an arriving ray’s power is received by the BS array with respect to the broadside.  At the base station the AoD and AoA have identical values.  The two values chosen to be in simulation are:

- AoD/AoA: 50 degrees (associated with the RMS Angle Spread of )

- AoD/AoA: 20 degrees (associated with the RMS Angle Spread of )

### BS Angle Spread

The base station per-path angle spread is defined as the root mean square (RMS) of angles with which a ray’s power is received or transmitted by the base station MEA. The individual path powers are defined in the temporal ITU SISO channel models. Three values of BS angle spread (each associated with a corresponding mean AoD/AoA) are considered in this document:

- AS: 2 degrees at AoD/AoA =

- AS: 5 degrees at AoD/AoA =

- AS: 25 degree  at AoD/AoA =  for the case of indoor pico cell

Attention should be paid when comparing the link level performance between the two angle spread values since the BS antenna gains for the two corresponding AoDs and AoAs are different.

### BS Power Azimuth Spectrum

The Power Azimuth Spectrum (PAS) of a ray arriving at the base station MEA exhibits Laplacian distribution. For an AoD  and RMS angle-spread , the BS per path PAS value at an angle  is given by:

where both angles and  are given with respect to the broadside of the MEA. It is assumed that all antenna elements’ orientations are aligned. Also, P is the average received power and G is the numeric base station antenna gain given by

Finally, No is the normalization constant:

In the above equation, represents path components (sub-rays) of the path power arriving at an AoD .

## Spatial Parameters for the Mobile Station

Calibrating MBWA simulators at link level requires the specification of a common set of spatial parameters for the mobile station.

### MS Antenna Topologies

At the MS, the MEA element spacing is 0.5, where is the wavelength of the carrier frequency.

For each antenna element at the MS, the antenna pattern will be assumed omni directional with an antenna gain of -1 dBi.

### MS Angle Spread

The MS per-path AS is defined as the root mean square (RMS) of angles of an incident path’s power at the MS array.  Two values of the path’s angle spread are considered:

- AS: 104o (results from the PAS with a uniform distribution over 360 degree),

- AS: 35o for a Laplacian PAS with a certain path specific Angle of Arrival (AoA).

### MS Angle of Arrival

The per-path Angle of Arrival (AoA) is defined as the mean angle of an incident ray at the MS MEA with respect to the broadside as shown in the figure below,

Figure 5.3.3-1 Angle of arrival orientation at the MS

The AoA analysis can provide an illustration of the PAS at MS MEA. Three different per-path AoA values at the MS are suggested for the cases of non-uniform PAS and uniform PAS,

- AoA: -67.5 degrees (associated with an RMS Angle Spread of 35o and 104o )

- AoA: +67.5 degrees (associated with an RMS Angle Spread of 35o and 104o )

- AoA: +22.5 degrees (associated with an RMS Angle Spread of 35o or with an LOS component)

### MS Power Azimuth Spectrum

The Power Azimuth Spectrum (PAS) of a ray arriving at the MS is modeled as either Laplacian distribution or uniform distribution over 360o. Since an omni antenna is assumed at MS, the received per path PAS will remain either Laplacian or uniform. For an incoming AoA and RMS angle spread, the MS per-path Laplacian PAS value at an angle  is given by:

where both angles and  are given with respect to the broadside of the MEA. It is assumed that all antenna elements’ orientations are aligned. Also, is the average received power and is the normalization constant:

.

In the above equation,  represents path components (sub-rays) of the path power arriving at an incoming AoA .

### MS Direction of Travel

The mobile station direction of travel is defined with respect to the broadside of the mobile antenna array as shown in the figure below,

Figure 5.3.5-1 Direction of Travel for MS

### Doppler Spectrum

The per-path Doppler Spectrum is defined as a function of DoT, per-path PAS, and AoA at MS. Doppler spectrum is affected by the PAS and the Angle of Arrival. Doppler spectrum affects the time-domain behavior of the channel.

## Definitions, Parameters, and Assumptions

The received signal at MS consists of N time-delayed multi-path replicas of the transmitted signal. These N paths are defined by the channel PDP, and are chosen randomly according to the channel generation procedure. Each path consists of M sub-paths. Figure 3.3 shows the angular parameters used in the model. The following definitions are used:

BS antenna array orientation, defined as the angle between the broadside of BS MEA and 	the absolute North (N) reference direction.

The angle between LOS direction and the broadside of BS array.

AoD for the nth ray with respect to the LOS, where (n = 1 … N).

Offset for the mth subpath of the nth ray with respect to, where (m = 1 … M).

Absolute AoD for the mth sub path of the nth ray at the BS with respect to the BS broadside.

MS antenna array orientation, defined as the angle between the broadside of the MS MEA and the 	absolute North reference direction.

Angle between the BS-MS LOS and the MS broadside

AoA of the nth ray with respect to LOS

Offset for the mth sub path of the nth ray with respect to.

Absolute AoA for the mth sub path of the nth ray at the MS w.r.t. the MS broadside

MS velocity vector

Angle of the velocity vector with respect to the MS broadside:

Note: The angle measured in a clockwise direction is assumed to be negative value.

Figure 5.4-1 The MIMO channel model angle parameters at both BS and MS

For the purpose of system level simulation, the fast fading per-path will be evolved in time, although bulk parameters including angle spread, delay spread, log normal shadowing, and MS location will remain fixed during its evaluation at one drop.

The following assumptions are made for all simulations, independent of environment:

- Uplink-Downlink Reciprocity: The AoD/AoA values are identical between the uplink and downlink.

- For FDD systems, random sub path phases between UL, DL are uncorrelated. (For TDD systems, the phases will be fully correlated.)

- Shadowing among different mobiles is uncorrelated. In practice, this assumption would not hold if mobiles are very close to each other, but we make this assumption just to simplify the model.

- The spatial channel model should allow any type of antenna configuration to be selected. In order to compare algorithms, reference antenna configurations based on Uniform Linear Array (ULA) configurations with 0.5, 4, and 10 wavelength inter-element spacing will be used.

- The composite AS, DS, and SF shadow fading, which may be correlated parameters depending on the channel scenario, are applied to all the sectors or antennas of a given base.  Sub-path phases are random between sectors.  The AS is composed of N x 20 sub-paths, and each sub-path has a precise AoD. The SF is a bulk parameter and is common among all the BS antennas or sectors.

- The elevation spread is not modeled here.

- To allow comparisons of different antenna scenarios, the transmit power of a single antenna case shall be the same as the total transmit power of a multiple antenna case.

- The generation of the channel coefficients assumes linear arrays. The procedure can be generalized for other array configurations.

## MIMO Channel Environments

The following channel environments will be considered for system level simulations.

- Suburban macro-cell

- Urban macro-cell

- Urban micro-cell

The table below describes the parameters used in each of the environments.

| Channel Scenario | Suburban Macro | Urban Macro | Urban Micro |  |
| --- | --- | --- | --- | --- |
| Number of paths (N) | 6 | 6, 11 | 6, 11 |  |
| Number of sub-paths (M) per-path | 20 | 20 | 20 |  |
| Mean AS at BS | E[σAS] =50 | E[σAS] =80, 150 | NLOS: E[σAS] =190 |  |
| AS at BS as a lognormal RV | μAS= 0.69 εAS= 0.13 | 80 μAS= 0.810 εAS= 0.34 150 μAS= 1.18 εAS= 0.210 | N/A |  |
| rAS=σAoDσAS | 1.2 | 1.3 | N/A |  |
| Per-path AS at BS (Fixed) | 20 | 20 | 50 (LOS and NLOS) |  |
| BS per-path AoD Distribution standard distribution | η(0,σAoD2)where σAoD=rASσAS | η(0,σAoD2)where σAoD=rASσAS | U(-40o, 40o) |  |
| Mean AS at MS | E[AS, MS] = 680 | E[AS, MS] = 680 | E[AS, MS] =680 |  |
| Per-path AS at MS (fixed) | 350 | 350 | 350 |  |
| MS Per-path AoA Distribution | η(0,σAoA2(Pr)) | η(0,σAoA2(Pr)) | η(0,σAoA2(Pr)) |  |
| Delay spread as a lognormal RV | DS = - 6.80 DS = 0.288 | DS = -6.18 DS = 0.18 | N/A |  |
| Mean total RMS Delay Spread | E[σDS] = 0.17 s | E[σDS] = 0.65 s | E[σDS] = 0.251s |  |
| rDS=σdelaysσDS | 1.4 | 1.7 | N/A |  |
| Distribution for path delays |  |  | U(0, 1.2s) |  |
| Lognormal shadowing standard deviation | 10dB | 10dB | NLOS: 10dB LOS: 4dB |  |
| Pathloss model (dB), d is in meters | 31.5 + 35log10(d) | 34.5 + 35log10(d) | NLOS:34.53+38log10(d) LOS:30.18 + 26*log10(d) |  |

Table 5.5-1 Environment Parameters

For calculations assume using a center frequency as a variable, for example path loss, 1900 MHz is assumed.

The following assumptions are made for the suburban macro-cell and urban macro-cell environments.

1.	The macrocell pathloss is based on the modified COST231 Hata urban propagation model:

where hbsis the BS antenna height in meters, hms the MS antenna height in meters, fcis the carrier frequency in MHz, d is the distance between the BS and MS in meters, and C is a constant factor (C = 0dB for suburban macro and C = 3dB for urban macro). Setting these parameters to hbs= 32m, hms= 1.5m, and fc=1900MHz, the path-losses for suburban and urban macro environments become, respectively, and . The distance d is required to be at least 35m.

2.	Antenna patterns at the BS are the same as those used in the link simulations.

3.	Site-to-site SF correlation is ζ=0.5.

4.	The hexagonal cell repeats will be the assumed layout.

The following assumptions are made for the micro-cell environment.

  - The microcell NLOS pathloss is based on the COST 231 Walfish-Ikegami NLOS model with the following parameters: BS antenna height 12.5m, building height 12m, building to building distance 50m, street width 25m, MS antenna height 1.5m, orientation 30deg for all paths, and selection of metropolitan center. With these parameters, the equation simplifies to:

PL(dB) = -55.9 + 38*log10(d) + (24.5 + 1.5*fc/925)*log10(fc).

The resulting pathloss at 1900 MHz is: PL(dB) = 34.53 + 38*log10(d), where d is in meters.  The distance d is at least 20m.  A bulk log normal shadowing applying to all sub-paths has a standard deviation of 10dB.

The microcell LOS pathloss is based on the COST 231 Walfish-Ikegami street canyon model with the same parameters as in the NLOS case. The pathloss is

PL(dB) = -35.4 + 26*log10(d) + 20*log10(fc)

The resulting pathloss at 1900 MHz is PL(dB) = 30.18 + 26*log10(d), where d is in meters.  The distance d is at least 20m.  A bulk log normal shadowing applying to all sub-paths has a standard deviation of 4dB.

  - Antenna patterns at the BS are the same as those used in the link simulations.

  - Site-to-site correlation is ζ=0.5.

  - The hexagonal cell repeats will be the assumed layout.

| Sub-path # (m) | Offset for a 2 deg AS at BS (Macrocell) Δn,m,AoD (degrees) | Offset for a 5 deg AS at BS (Microcell) Δn,m,AoD(degrees) | Offset for a 35 deg AS at MS Δn,m,AoA(degrees) |
| --- | --- | --- | --- |
| 1, 2 | ±0.0894 | ±0.2236 | ±1.5649 |
| 3, 4 | ±0.2826 | ±0.7064 | ±4.9447 |
| 5, 6 | ±0.4984 | ±1.2461 | ±8.7224 |
| 7, 8 | ±0.7431 | ±1.8578 | ±13.0045 |
| 9, 10 | ±1.0257 | ±2.5642 | ±17.9492 |
| 11, 12 | ±1.3594 | ±3.3986 | ±23.7899 |
| 13, 14 | ±1.7688 | ±4.4220 | ±30.9538 |
| 15, 16 | ±2.2961 | ±5.7403 | ±40.1824 |
| 17, 18 | ±3.0389 | ±7.5974 | ±53.1816 |
| 19, 20 | ±4.3101 | ±10.7753 | ±75.4274 |

Table 5.5-2 Sub-path AoD and AoA offsets

The values in above table are selected to produce a biased standard deviation equal to 2, 5, and 35 degrees, which is equivalent to the per-path power weighted azimuth spread for equal power sub-paths.

# Procedure for Generating SCM Parameters

## Introduction

For a given scenario and a set of parameters, realizations of each channel model parameters such as the path delays, powers, and sub-path AoD and AoA can be derived by using the procedure described in this section. In particular, Section 6.1.1 describes the steps for the urban macrocell and suburban macrocell environments, and Section 6.1.2 describes the steps for the urban microcell environment. For detailed information, please see reference [13].

### Generating Model Parameters for Urban and Suburban Macrocell Environments

Step 1: Choose either an urban macrocell or suburban macrocell environment.

Step 2: Determine various distance and orientation parameters.

Step 3: Determine the DS, AS, and SF.

Step 4: Determine random delays for each of the N multipath components.

Step 5: Determine random average powers for each of the N multipath components.

Step 6: Determine  AoDs for each of the N multipath components.

Step 7: Associate the multipath delays with AoDs.

Step 8: Determine the powers, phases and offset AoDs of the M = 20 sub-paths for each of the N paths at the BS.

Step 9: Determine the AoAs for each of the multipath components.

Step 10: Determine the offset AoAs at the UE of the M = 20 sub-paths for each of the N paths at the MS.

Step 11: Associate the BS and MS paths and sub-paths.

Step 12: Determine the antenna gains of the BS and MS sub-paths as a function of their respective sub-path AoDs and AoAs.

Step 13: Apply the path loss based on the BS to MS distance from Step 2, and the log normal shadow fading determined in step 3 as bulk parameters to each of the sub-path powers of the channel model.

### Generating Model Parameters for Urban Microcell Environments

Urban microcell environments differ from the macrocell environments in that the individual multipaths are independently shadowed.]

Step 1: Choose the urban microcell environment.

Step 2: Determine various distance and orientation parameters.

Step 3: Determine the bulk path loss and log normal shadow fading parameters.

Step 4: Determine the random delays for each of the N multipath components.

Step 5: Determine random average powers for each of the N multipath components.

Step 6: Determine  AoDs for each of the N multipath components.

Step 7: Randomly associate the multipath delays with AoDs.

Step 8: Determine the powers, phases, and offset AoDs of the M = 20 sub-paths for each of the N paths at the BS.

Step 9: Determine the AoAs for each of the multipath components.

Step 10: Determine the offset AoAs of the M = 20 sub-paths for each of the N paths at the MS.

Step 11: Associate the BS and MS paths and sub-paths.  Sub-paths are randomly paired for each path, and the sub-path phases defined at the BS and MS are maintained.

Step 12: Determine the antenna gains of the BS and MS sub-paths as a function of their respective sub-path AoDs and AoAs.

Step 13: Apply the path loss based on the BS to MS distance and the log normal shadow fading determined in Step 3 as bulk parameters to each of the sub-path powers of the channel model.

## Generating SCM Coefficients

Given the user parameters generated in Section 6.1, we use them to generate the channel coefficients. For an S element linear BS array and a U element linear MS array, the channel coefficients for one of N multipath components are given by a  matrix of complex amplitudes. We denote the channel matrix for the nth multipath component (n = 1,…,N) as Hn(t). The (u,s)th component (s = 1,…,S; u = 1,…,U) of Hn(t) is given by

where

Pn		is the power of the nth path (Step 5).

σSF	is the lognormal shadow fading (Step 3), applied as a bulk parameter to the n paths for a given drop.

M		is the number of subpaths per-path.

θn,m,AoD	is the AoD for the mth subpath of the nth path (Step 12).

θn,m,AoA		is the  AoA for the mth subpath of the nth path (Step 12).

GBS(θn,m,AoD)	is the BS antenna gain of each array element (Step 12).

GMS(θn,m,AoA)	is the MS antenna gain of each array element (Step 12).

j	is the square root of -1.

k		is the wave number 2πλwhere λis the carrier wavelength in meters.

ds	is the distance in meters from BS antenna element s from the reference (s = 1) antenna. For the reference antenna s = 1, d1=0.

du	is the distance in meters from MS antenna element u from the reference (u = 1) antenna. For the reference antenna u = 1, d1=0.

Φn,m	is the phase of the mth subpath of the nth path (Step 8)

v	is the magnitude of the MS velocity vector (Step 2)

θv	is the angle of the MS velocity vector (Step 2)

The path loss and the log normal shadowing is applied as bulk parameters to each of the sub-path components of the n path components of the channel.

# A Method of Generating Spatial Correlation Coefficients for MIMO Channels

Performance of MIMO systems in realistic radio environments greatly depends on the spatial correlation due to the presence of scatterers in the propagation environments. In order to predict the performance of MBWA system in real environments by simulation, we develop the correlated MIMO fading channel model by adopting the available stochastic spatial parameters that have been identified as the best fit to the propagation measurements. Cross correlation between radio waves arriving at two different antenna elements is a function of PAS (Power Azimuth Spectrum), radiation pattern of antenna and antenna element spacing.

We assume mutipath signals are represented by channel taps each of which is represented a cluster of scatterers. MIMO channel matrix generated by on a tap by tap basis can be written as                                                Hi=Rr12HiRt12

where Hi is an N×M  matrix of  iid complex Gaussian random variables with zero mean, unit variance. Rrand Rtare the correlation matrices at the receiver and transmitter.

Received signal at the mth array element at time t can be written as

rm(t)=s(t)1Pi=1Pgi(t)ejDmsin(φ0−φi)

where P is the number of sub-paths per channel for a given channel tap, s(t) is the complex envelope, g(t) the random fading coefficient for this sub-path, and D=2πdλnormalized distance between array elements.

Assuming the angles of arrival (AoA)s to be independent across different sub-paths, cross correlation between the mth and nth array element can be evaluated as

Rm,n=Ermrn=  Rm,n=EφejD(m−n)sin(φ0−φ)

Assuming antenna pattern of unity,

Rm,n=−π+πejD(m−n)sin(φ0−φ)Pφdφ

Stochastical Modeling of Experimental Test bed measurements of outdoor propagation environments concluded that Lapacain function accurately describes the estimated PAS [28]. We define truncated Laplacian distribution of PAS is defined [31] as under:

Pφ=c2σφ.e−2φ−φ0σφ−Δ+φ0φ≤Δ+φ0

c is a normalization constant which guarantees that the integration Pφ over the range [−Δ+φ0,Δ+φ0] is unity.  This distribution can be also obtained from the  Laplacian distribution defined in Section 6 by assuming that the incoming rays are within ±Δof the mean angle of arrival and that G(θ)=1.

An exact expression for Spatial Correlation coefficients is given [28] as sum of Bessel functions of the first kind,

ℜ[R(φ0,σφ)]m,n=J0Dm−n+2r=1∞J2rDm−n2σφ22+2r2cos2rφ02σφ2+exp−Δ2σφ22rsin2rΔ−2σφ2cos2rΔℑ[R(φ0,σφ)]m,n=2r=1∞J2r+1Dm−n2σφ22+2r+12sin2r+1φ02σφ2−exp−Δ2σφ22r+1sin2r+1Δ+2σφ2cos2r+1Δ

Where φ0,Δ and σφ are the parameters for Laplacian distributed PAS as defined in (23).

In the above equation, φ represents path components (sub-rays) of the path power arriving at an AoA φ0.

When Δ is very small, Laplacian distribution is defined over the range (-π,π), spatial correlation between mth and nth antenna element at the same antenna array is approximated as [29,30],

[R(φ0,σφ)]m,n≈cejD(m−n)sin(φ0)[B(φ0,σφ)]m,n

where

[B(φ0,σφ)]m,n=11+σφ22[D(m−n)cosφ0]2

# References and Bibliography

[1] C802.20-03/50, “Overview of METRA Model for MBWA MIMO Channel”, IEEE 802.20 Session #2

[2] C802.20-03/49, “Comparison of SFBC and STBC for Transmit Diversity in OFDM System”, IEEE 802.20 Session #2

[3] C802.20-03/48, “Channel Models and Performance Implications for OFDM-based MBWA”, IEEE 802.20 Session #2

[4] C802.20-03/46r1, “Channel Requirements For MBWA (Rev 1)”, IEEE 802.20 Session#2.

[5] C802.20-03/43, “802.20 Evaluation Methodology Strawman”, IEEE 802.20 Session #2

[6] C802.20-03/42, “Channel Modeling for MBWA”, IEEE 802.20 Session#2.

[7] C802.20-03/35, “Evaluation Methodology for MBWA”, IEEE 802.20 Session #2.

[8] C802.20-03/18, “MIMO Channel Model for MBWA”, IEEE 802.20 Session #1.

[9] C802.20-03/15r1, “Channel Models and Performance Implications for OFDM-based MBWA”, IEEE 802.20 Session #1.

[10] C802.20-03/12, “Antenna Arrays for MBWA: Overview and Field Experiments”, IEEE 802.20 Session#1.

[11] C802.20-03/09, “Channel Modeling Suitable for MBWA”, IEEE 802.20 Session #0.

[12] 3GPP TS 45.005 V5.9.0, “3GPP: Technical Specification Group GSM/EDGE Radio Access Network; Radio transmission and reception (Release 1999).

[13] 3GPP & 3GPP2 Spatial Channel Model AHG, “Spatial Channel Model Text Description”, SCM Text V7.0.

[14] ETSI TR 101 112, UMTS 30.03, V3.2.0, Annex B, Sections 1.2.3, 1.3, 1.4.

[15] IEEE 802.16.3c-01/29r4, “Channel Models for Fixed Wireless Applications”, 2001-07-17.

[16] IEEE 802.16.3c-00/49r2, “Interim Channel Models for G2 MMDS Fixed Wireless Applications”, 2000-11-15.

[17] C802.20-03/77, “Summary of Delay Profiles for MBWA”, IEEE 802.20 Session #4.

[18] I.E. Telatar, “Capacity of Multi-Antenna Gaussian Channels”, Technical Report, AT&T Bell Labs., 1995.

[19] K. I. Pedersen, J. B. Andersen, J. P. Kermoal, and P.E. Mogensen, “A Stochastic Multiple-Input-Multiple-Output Radio Channel Model for Evaluation of Space-Time Coding Algorithms”, VTC 2000 Fall, Boston, MA, Sept. 2000, pp. 893-897.

[20] J.P. Kermoal, L. Schumacher, K. I. Pedersen, P.E. Mogensen, and F. Frederiksen,“A Stochastic MIMO Radio Channel Model with Experimental Validation”, IEEE JSAC, V.20, No.6, pp.1211-1226, August 2002.

[21] 3GPP2 TSG-C R1002, “1xEV_DV Evaluation Methodology (V13)”, 2003.

[22] K.I. Pedersen, P.E. Mogensen, and B.H. Fleury, “Spatial Channel Characteristics in Outdoor Environments and their Impact on BS Antenna System Performance”, VTC’98, Ottawa, Canada, pp. 719-723, May 1998.

[23] L. Greenstein, V. Erceg, Y.S. Yeh, M.V. Clark, “A New Path-Gain/Delay-Spread Propagation Model for Digital Cellular Channels”, IEEE Transactions on Vehicular Technology, Vol. 46, No. 2, May 1997, pp. 477-485.

[24] IEEE C802.20-04-79, “Overview of the Spatial Channel Model developed in 3GPP-3GPP2”.

[25] ITU-R RECOMMENDATION M.1225, "GUIDELINES  FOR  EVALUATION  OF  RADIO  TRANSMISSION TECHNOLOGIES  FOR  IMT-2000"

[26]   CR802.20-05-32r1 “Channel Modeling for MBWA”

[27] A. F. Naguib, “Adaptive Antennas for CDMA Wireless Networks”, Ph.D. Thesis, Stanford University, Stanford, CA 1996.

[28] Schumacher, L.; Pedersen, K.I.; Mogensen, P.E.; “From antenna spacings to theoretical capacities - guidelines for simulating MIMO systems”, The 13th IEEE International Symposium on Personal, Indoor and Mobile Radio Communications, 2002. Volume 2, 15-18 Sept. 2002 Page(s):587 - 592 vol.2

[29] A.Forenza, D.J.Love, and R.W.Heath Jr., “A low complexity algorithm to simulate the Spatial Covariance Matrix for clustered MIMO Channel Models, IEEE Vehicular Technology Conference, 2004.

[30] A.Forenza, D.J.Love, and R.W.Heath Jr., “Simulation of the Spatial Covariance Matrix”, doc.: IEEE 802. 11-03/925r0, 802.11 TGnChannel Model Special Committee,Nov.2003

[31] IEEE C802.20-05/37

[32] IEEE  C802.20-05/38