----- HELP------

Using The Program

CALCULATE - After entering the Frequency in KZ Then Click (CALCULATE).

PRINT    -After seeing the antenna lengths on the screen, If you want A printout just Click On PRINT.

HELP     - This Is the Help File You Are Looking At Now. To Return To the program just Click this screen.

QUIT     - Clicking ON the Quit Button Will END the Program.

-- CLICK ON THIS SCREEN TO RETURN TO PROGRAM.----

(1) START USING THE PROGRAM BY TYPING IN THE FREQUENCY OF THE ANTENNA YOU

WANT TO BUILD.

(2) CLICK THE "CALCULATE"  BUTTON AND YOUR MEASURMENTS COME ON THE SCREEN.

(3) TO GET HARD COPY OF THE MEASURMENTS CLICK THE "PRINT"  BUTTON.

(4) TO QUIT CLICK THE "QUIT" BUTTON.

___________________________________

BUILDING THE ANTENNA

WHEN THE HELP SCREEN COMES UP YOU WILL SEE A PICTURE OF THE CENTER

PART OF A DOUBLE BAZOOKA.

MEASURE THE COAX TO THE LENGTH THE PROGRAM SAYS FOR CENTER OF COAX

CUT THE BLACK COVER OFF ABOUT 3 INCHES EACH SIDE OF CENTER.

CUT THE SHIELD IN THE CENTER WITH A SHARP KNIFE.

TAKE A POINTED TOOL AND UNWEAVE THE 3 INCH BRAIDS ON EACH SIDE.

CONNECT THE SHIELD OF YOUR COAX FEEDLINE TO ONE OF THE SHIELDS OF THE ANTENNA.

CONNECT THE CENTER OF YOUR COAX FEEDLINE TO THE OTHER SHIELD ON THE ANTENNA.

TAPE THIS CONNECTION UP REAL GOOD TO KEEP WATER OUT.

IF THIS ANTENNA IS GOING TO BE SUPORTED ON THE ENDS. DON'T USE RG8 AND

MAKE A STRAIN RELIEVE BETWEEN THE FEED LINE AND THE ANTENNA.

________________________

ENDS OF COAX

REMOVE THE BLACK COVER ON EACH END OF YOUR COAX ABOUT 2 INCHES.

REMOVE THE INSULATING MATERIAL BETWEEN BRAID AND CENTER CONDUCTOR.

UNWEAVE THE SHIELD LIKE YOU DID THE CENTER PIECE.

TWIST THE CENTER CONDUCTOR AND SHIELD TOGETHER AND SOLDER.

CONNECT ONE OF THE END WIRES TO THIS SAME CONNECTION AND SOLDER ALL

THREE TOGETHER.

REMEMBER THAT THEY WILL HAVE ALOT OF STRAIN ON THESE CONNECTIONS SO PUT

THEM TOGETHER SO THE STRAIN IS NOT ON THE SOLDER.

______________________________

THIS IS A FREE PROGRAM TO ANYONE WHO WANTS IT. BILL W4BEJ

MY E-MAIL ADDRESS IS w4bej@ecc-uky.campus.mci.net