Patrick LINDECKER (F6CTE) the 19th of september 2003

SPECIFICATIONS OF THE PSK10 MODE (MULTIPSK version 2.4)

GENERAL SPECIFICATIONS

Creator: Patrick Lindecker F6CTE (2001)

Description :

Baud rate 	: 10

Speed 	: 18 wpm

Modulation 	: DBPSK

Reception mode: indifferent (LSB or USB)

Character set  	: 56 "Varicode" characters " A..Z 0..9    <CR> <ESPACE> . , " : - = + ? $ ' ( ) ! / @ & # * with an error reset character and an idle character (displayed by ">").

Small characters are transformed in capital characters and special characters as

" é " are transformed in standard capital (in the example: " E ").

Shape of pulse 	: rectangular but the connection between 2 successive different bits is done by a 			decreasing sine then by a creasing sine (each of one during quarter of period)

Bandwidth 	: about 40 Hz

Demodulation 	: coherent,

Synchronization: automatic using the signal

Correction code: no

Convolution code: no

Interleaving	: no

Repetition 	: no

Drift tolerance 	: 40 Hz/mn (depending on signal level)

Lowest S/N for 2% errors: -17 dB (for white noise in a 3 kHz bandwidth)

CHOICE OF THE COMMUNICATION MODE

The most simple is a two phases modulation in a biorthonormal form: 0° phase and 180° phase (inverse). It must be seen as BPSK (binary PSK) or binary PAM. It is also a very efficient mode to transmit in white noise with a low probability of symbol error.

To avoid any phase reference, it will be chosen a differential mode (what is detected is not the phase but its inversion or not: 0°--->180 ° or 180°--->0° for inversion and 0°--->0 ° or 180°--->180° for non-inversion). Unfortunately, the bit error rate is doubled.

The detection remains coherent (in fact “ quasi-coherent ” because the synchronization is not sure but built with the signal).

Note: it’s called “coherent” because a PLL is used to follow the phase. But the phase is known with an ambiguity of 180 °, so it’s not a real “coherent” detection.

The transmission may be done in LSB or USB. But to simplify, USB is recommended so when a frequency is proposed everybody speak of the same QRG.

The transmission is done in 10 baud’s, D-2PSK (Differential two-phases shift keying).

10 baud’s (0.1 sec by bit) because this corresponds to 18 words/min. in CW which is the minimum speed for a normal talk and because a frequency of 10 Hz would be easier to realize with a frequency reference based on a “second clock” (as transmitted by radio clocks, for example).

SHAPE FOR PULSE

The type of filter used for sending bits has been chosen after some experimentations.

I finally choose a special shape of pulse: rectangular but the connection between 2 successive different bits is done by a decreasing sine then by a creasing sine (each of one during a quarter of period). This allows a not too large bandwidth without interference between symbols. Except during transitions, the envelope of the level is constant so the average power is 85 % of the maximum power (obtained for example with a “tune”).

Example: 1011:   1    |   0   |  1    |   1

R-R-R-D-C-R-R-D-C-R-R-R-R-R-R-R

or C                          or D

R: rectangular (=1) of ¼ T. D: Sine decreasing of ¼ T (1 to 0)

C: Sine creasing of ¼ T (0 to 1)

BITS DISPOSITION AND SET OF CHARACTERS

For bits disposition, PSK10 set of characters is inspired from PSK31 of Peter Martinez (Varicode) with a separation code composed with “011” (“1” for “state change” and “0” for “no change”).

"011" is also the idle character.

The reception of phase changes is done from left to right.

The set of characters is more concise than the one of PSK31 and is inspired from RTTY Baudot for three reasons:

* to have not to longs sequences of bits,

* due to the weak ratio Signal-to-Noise to aim, the synchronization imposes a maximum of transitions,

* to make possible corrections of bits when character is not found or if it’s not in the character set. In general, only one bit is affected. With this hypothesis it’s possible to detect with a good probability which bit is false. That helps a bit when S/N ratio becomes bad.

| CHARACTER | CODE |
| --- | --- |
| Idle character (>) |  |
| Space | 1 |
| E | 0 |
| T | 1 1 |
| A | 0 1 |
| I | 1 0 |
| S | 0 0 |
| N | 1 1 1 |
| R | 1 0 1 |
| O | 1 1 0 |
| <CR>+<CL> | 0 0 1 |
| C | 0 1 0 |
| D | 1 0 0 |
| L | 1 1 1 1 |
| P | 1 1 0 1 |
| M | 1 1 1 0 |
| H | 0 1 0 1 |
| U | 1 0 0 1 |
| F | 1 0 1 0 |
| B | 1 1 0 0 |
| G | 0 0 1 0 |
| 0 | 1 1 1 1 1 |
| 9 | 1 1 1 0 1 |
| 5 | 1 1 1 1 0 |
| 1 | 1 0 1 0 1 |
| 2 | 1 1 0 0 1 |
| 3 | 1 1 0 1 0 |
| 4 | 1 1 1 0 0 |
| 6 | 0 1 0 0 1 |
| 7 | 0 1 0 1 0 |
| 8 | 1 0 0 1 0 |
| . | 1 1 1 1 1 1 |
| Y | 1 1 1 1 0 1 |
| W | 1 1 1 1 1 0 |
| V | 1 1 0 1 0 1 |
| , | 1 1 1 0 0 1 |
| " | 1 1 1 0 1 0 |
| X | 1 1 1 1 0 0 |
| K | 0 1 0 1 0 1 |
| Error reset key (<) | 1 0 0 1 0 1 |
| : | 1 0 1 0 0 1 |
| - | 1 0 1 0 1 0 |
| = | 1 1 0 0 1 0 |
| + | 0 0 1 0 0 1 |
| ? | 0 0 1 0 1 0 |
| Q | 0 1 0 0 1 0 |
| $ | 1 1 1 1 1 1 1 |
| ' | 1 1 1 1 1 0 1 |
| Z | 1 1 1 1 1 1 0 |
| J | 1 1 1 0 1 0 1 |
| ( | 1 1 1 1 0 0 1 |
| ) | 1 1 1 1 0 1 0 |
| ! | 1 1 1 1 1 0 0 |
| / | 1 0 1 0 1 0 1 |
| @ | 1 1 0 0 1 0 1 |
| & | 1 1 0 1 0 0 1 |
| # | 1 1 0 1 0 1 0 |
| * | 1 1 1 0 0 1 0 |

The small letters keystroke at the editor are translated in capital letters.

IMPORTANT: in the same order, all the accentuated letters are translated in the equivalent non-accentuated letter. For example,    é and É become E.

Precisely, here is a part of the program (PASCAL) for reception of characters:

CARAC_RECU is the received character, REUSSITE the flag to specify if the

character is from PSK10 character and CARAC_A_AFFICHER is the character to display.

PROCEDURE VERIF_CARACTERE_ET_TRADUCTION_PSK10(CARAC_RECU:CHAR;

VAR REUSSITE:BOOLEAN;VAR CARAC_A_AFFICHER:CHAR);

BEGIN

REUSSITE:=FALSE;

CARAC_A_AFFICHER:=CHR(0);

{a...z}

IF (CARAC_RECU>='a') AND (CARAC_RECU<='z') THEN

BEGIN

REUSSITE:=TRUE;

CARAC_A_AFFICHER:=CHR(ORD(CARAC_RECU)-32);

EXIT;

END;

IF (CARAC_RECU='Ç') OR (CARAC_RECU='ç') THEN CARAC_RECU:='C';

IF (CARAC_RECU='ü') OR (CARAC_RECU='û') OR (CARAC_RECU='ù') OR

(CARAC_RECU='Ü') OR (CARAC_RECU='ú')    THEN CARAC_RECU:='U';

IF (CARAC_RECU='é') OR (CARAC_RECU='ê') OR

(CARAC_RECU='æ') OR (CARAC_RECU='Æ') OR (CARAC_RECU='ë') OR

(CARAC_RECU='è') OR (CARAC_RECU='É')    THEN CARAC_RECU:='E';

IF (CARAC_RECU='â') OR (CARAC_RECU='ä') OR (CARAC_RECU='à') OR

(CARAC_RECU='å') OR (CARAC_RECU='Ä') OR (CARAC_RECU='Å')

OR (CARAC_RECU='á')    OR (CARAC_RECU='ª') THEN CARAC_RECU:='A';

IF (CARAC_RECU='ï') OR (CARAC_RECU='î') OR (CARAC_RECU='ì') OR

(CARAC_RECU='í') THEN CARAC_RECU:='I';

IF (CARAC_RECU='ô') OR (CARAC_RECU='ö') OR (CARAC_RECU='ò') OR

(CARAC_RECU='Ö') OR (CARAC_RECU='ó') OR (CARAC_RECU='º')    THEN CARAC_RECU:='O';

IF (CARAC_RECU='ô') OR (CARAC_RECU='ö') OR (CARAC_RECU='ò') OR

(CARAC_RECU='Ö') OR (CARAC_RECU='ó')    THEN CARAC_RECU:='O';

IF (CARAC_RECU='ÿ') THEN CARAC_RECU:='Y';

IF (CARAC_RECU='ø') OR (CARAC_RECU='Ø') THEN CARAC_RECU:='0';

IF (CARAC_RECU='×') THEN CARAC_RECU:='X';

IF (CARAC_RECU='ƒ') THEN CARAC_RECU:='F';

IF (CARAC_RECU='¿') THEN CARAC_RECU:='?';

IF (CARAC_RECU='©')OR (CARAC_RECU='¢') THEN CARAC_RECU:='C';

IF (CARAC_RECU='Ð') THEN CARAC_RECU:='D';

IF (CARAC_RECU='ñ') OR (CARAC_RECU='Ñ') THEN CARAC_RECU:='N';

IF ((CARAC_RECU>='A') AND (CARAC_RECU<='Z')) OR

((CARAC_RECU>='0') AND (CARAC_RECU<='9')) OR

(CARAC_RECU='.') OR (CARAC_RECU=',') OR    (CARAC_RECU='"') OR

(CARAC_RECU=':') OR (CARAC_RECU='-') OR    (CARAC_RECU='=') OR

(CARAC_RECU='+') OR (CARAC_RECU='?') OR    (CARAC_RECU='''') OR

(CARAC_RECU='(') OR (CARAC_RECU=')') OR    (CARAC_RECU='!') OR

(CARAC_RECU='/') OR (CARAC_RECU='@') OR    (CARAC_RECU='*') OR

(CARAC_RECU='#') OR (CARAC_RECU='&') OR (CARAC_RECU='$') OR

(CARAC_RECU=' ') OR (CARAC_RECU=CHR(13)) OR (CARAC_RECU='%') THEN

BEGIN

REUSSITE:=TRUE;

CARAC_A_AFFICHER:=CARAC_RECU;

END;

END;

The idle character (">") is transparent for the user. The Error reset ("<") key corresponds to the "Back Space".