Patrick LINDECKER (F6CTE) the 22th of july 2003

SPECIFICATIONS OF THE PSKFEC31 MODE (MULTIPSK version 2.3)

Digital communications are, sometimes, given impossible, because of:

* QRM of others radio-stations,

* fading,

* a too weak ratio Signal-to-Noise ratio (the signal is drowned in the noise) ,

* ionospheric Doppler modulation (for low baud speed) or time overlap between different paths (for quick baud speed)…

The aim of author, for PSKFEC31, is to allow communications in following conditions:

* “chat” communication between ham people (no file transfer) in short waves with much less errors that with PSK31,

* possible communication on a AF spectrum from 200 to 3000 Hz (transmission band of a SSB receiver), with less than 2 % of false received characters, this for a Signal-to-Noise ratio (before filter) inferior or equal to the one obtained for PSK31 (-11,5 dB: signal 14 times less powerful than noise).

* possible communication under SW bad conditions,

* speed of transmission about 30 words/minute, in equivalence with CW, this speed corresponds to a quick typing for an OM,

* transceiver stable in frequency (same stability as for PSK31),

* PC available for the OM: Pentium 166 or more with a sound card (SOUND BLASTER compatible).

Comparison with BPSK31: it is reminded that the BPSK31 mode is designed to assure communications with ratio Signal-to-Noise until -11.5 dB. The equivalent Morse speed is 37 words/minute in capital and 51 words/minute in small letters.

The difference between the PSK31 (-11.5 dB) and PSKFEC31 (-13.5 dB) is about 2 dB in Signal-to-Noise ratio.

GENERAL SPECIFICATIONS

Creator: Patrick Lindecker F6CTE (06/2003)

Description :

Baud rate 	: 31.25

Speed 	: 28 wpm

Modulation 	: DBPSK

Reception mode: indifferent (LSB or USB)

Character set 	: 56 "Varicode" characters " A..Z 0..9    <CR> <ESPACE> . , " : - = + ? $ ' ( ) ! / @ & # * with an error reset character and an idle character (displayed by ">").

Small characters are transformed in capital characters and special characters as

" é " are transformed in standard capital (in the example: " E ").

Shape of pulse 	: rectangular but the connection between 2 successive different bits is done by a 			decreasing sine then by a creasing sine (each of one during quarter of period)

Bandwidth 	: about 160 Hz

Demodulation 	: coherent,

Synchronization: automatic using the signal

Correction code: no

Convolution code: no

Interleaving	: no

Repetition 	: each bit is repeated 13 positions later (so 0.42 s between 2 transmissions of the same bit). This diversity in time allows the correction of an erroneous bit. The synchronization is easy.

Automatic Frequency Correction: from +/-3 Hz to +/- 10 Hz depending on signal level,

Drift tolerance 	: 60 Hz/mn for PSKFEC31 (depending on signal level)

Lowest S/N for 2% errors: -13.5 dB (for white noise in a 3 kHz bandwidth)

Due to the FEC process, this mode permits to reduce appreciably the number of errors due to SW radio conditions, compared to BPSK31.

CHOICE OF THE COMMUNICATION MODE

First it must be determined what communication mode to choose, the most interesting with regards with the preceding hypothesis.

The most simple is a two phases modulation in a biorthonormal form: 0° phase and 180° phase (inverse). It must be seen as BPSK (binary PSK) or binary PAM. It is also a very good mode to transmit in white noise with a low probability of symbol error.

To avoid any phase reference, it will be chosen a differential mode (what is detected is not the phase but its inversion or not: 0°--->180 ° or 180°--->0° for inversion and 0°--->0 ° or 180°--->180° for non-inversion). Unfortunately, the bit error rate is doubled.

The detection remains coherent (in fact “ quasi-coherent ” because the synchronization is not sure but built with the signal).

Note: it’s called “coherent” because a PLL (Costas loop) is used to follow the phase. But the phase is known with an ambiguity of 180 °, so it’s not a real “coherent” detection.

###### A LITTLE HISTORY

A first mode was proposed which name was PSK10: the transmission was done in 10 baud’s, D-2PSK (Differential two-phases shift keying). A Varicode character set was associated to this mode. 10 baud’s (0.1 sec by bit) because this corresponds to 20 words/min in CW which is the minimum speed for a normal talk and because a frequency of 10 Hz would be easier to realize with a frequency reference based on a “second clock” (as transmitted by radio clocks, for example). But without correction, the duration of the bit (0,1 second) is too long in front of the ionospheric Doppler modulation in SW.

The time overlap between different paths makes problem for 100 bauds mode and above. So, for the SW, a reasonable speed is about 30/60 bauds.

To reduce the errors rate, I propose a solution which consists to repete each character (PSKAM10/31/50). The goal was reached. But, under bad conditions (QRM for example), there was a problem of loss of synchronisation. As it is difficult to synchronize under these bad conditions (a character is 8 bits long so there are 16 possibilities...), the transmission might be good but only by slices. Nevertheless, the surely gain of PSKAM is that, when synchronized, there are no erroneous characters due to noise.

###### APPLICATION TO PSKFEC31

I choose a PSK10 set of characters for two reasons:

* to have a concise set of characters which reduce the errors rate (the more the choice is big and the more you have probability to produce an error),

* to have a normal speed for a ham (28 words per minute for 31.25 bauds). With a PSK31 set of characters, the speed would be about 23 words per minute.

The modulation speed (31,25 bauds) is chosen to avoid PSK10 problem (duration of the bit too long in front of the ionospheric Doppler modulation in SW) and to be consistent with PSK31.

To avoid the synchronization problem of PSKAM, I repete the bits and not the characters, so the synchronization is much more easy (choise between 2 possibilities).

The type of windowing filter used for sending bits has been chosen after some experimentations (same as PSK10 and PSKAM10/50).

I finally choose a special shape of pulse: rectangular but the connection between 2 successive different bits is done by a decreasing sine then by a creasing sine (each of one during a quarter of period). This allows a not too large bandwidth without interference between symbols. Except during transitions, the envelope of the level is constant so the average power is 85 % of the maximum power (obtained for example with a “tune”).

Note: this is not critical and a "raised cosinus" as in BPSK31 could have been chosen.

Example: 1011:   1    |   0   |  1    |   1

R-R-R-D-C-R-R-D-C-R-R-R-R-R-R-R

or C                          or D

R: rectangular (=1) of ¼ T. D: Sine decreasing of ¼ T (1 to 0)

C: Sine creasing of ¼ T (0 to 1)

The transmission may be done in LSB or USB. But to simplify, USB is recommended so when a frequency is proposed everybody speak of the same QRG.

BITS DISPOSITION AND SET OF CHARACTERS

PSK10 set of characters has been chosen so refer to PSK10.DOC for description of this set of characters.

# DESCRIPTION OF THE PROTOCOL

##### THE STATION SENDING

Each bit is repeated 13 positions later, with the same principle as in AMTOR FEC except that it is related to bit and not to character.

Suppose a "phase change bit" noted "1" and a "no phase change bit" noted "0". Suppose that instead of a difference of 13 bitss we had a difference of 5 bits. We want to transmit 101110:

DX 		RX			DX is the first position

1		x1			RX is the second position

0		x2	 		x1, x2 are the preceding bits (0 or 1)

1		1 			y1, y2 are following bits (0 or 1)

1		0

1		1

0		1

y1		1

y2		0

It will be transmitted (DX then RX):

1 x1 O x2 1 1 1 0 1 1 0 1 y1 1 y2 0

##### THE STATION RECEIVING

At the reception, the DX and RX positions are unknown. As there are two sequences of bits possible, it is easy to find the good one making an autocorrelation from these sequences (on two seconds duration for example) and choosing the one with the largest autocorrelation.

Example:

I receive: 1 x1 O x2 1 1 1 0 1 1 0 1 y1 1 y2 0

2 sequences are possible: starting with the first "1" or starting with the x1 bit.

Sequence with the first "1" (on 5 bits):

1/1,    0/0, 1/1, 1/1, 1/1, 0/0

Sequence with x1 (on 5 bits):

x1/1, x2/1, 1/0, 0/y1, 1/y2

As x1, x2, y1,y2 are worth 0 or 1, the autocorreleration will be maximum with the first sequence so the first "1" will be the DX position.

The determination of the DX position will be done regurarly (two times per second for example).

Once synchronized, the more probable bit is the mean bit:

Mean bit = (DX bit +RX bit) / 2.

Suppose I must receive a "1". In term of soft decesion, I must receive 1000 for "1" (it would be -1000 for a "0"). I note that the received DX bit is worth -100 (instead of 1000, so I could would conclude for a "0") and the received RX is worth 700 (instead of 1000 but I would conclude for a "1"). There is an ambiguity. The mean bit is worth 300, so the decision would be "1" and it would be the good one.

In this example, the false DX bit is corrected by the good RX bit. So the error rate is reduced by a factor from 2 to 10 depending on the conditions (QRM, QSB, ionospheric Doppler modulation...). By correlation, this mode is less sensitive to bad S/N ratio.

Note: this process could possibly be done in term of angles if the angle was calculated (method not applied by the author).

DETECTION OF SIGNAL IN NOISE AND DISPLAY

One difficulty is to detect and display a very weak signal in noise. The only way to do it, is to average the spectrums. For example, in MULTIPSK, the displayed spectrum for PSKFEC31 is an average of 3 spectrums (3 by second). As a signal (as PSKFEC31) is coherent and noise is non-coherent, the average of noise becomes nil and the average of the signal remains constant, so the more the number of spectrums averaged is, the better the signal comes out from noise. The problem is in the introduction of a time delay (the averaging is a filter) so the phenomena’s becomes slow…the increasing and the decreasing of the signals are slow.

In conclusion, it’s a compromise between fine detection and time delay…and it’s one of the limit for detection of signal in noise: to pass to a limit of S/N ratio of 0.001 (-30 dB) for example, it will be necessary to have a baud rate of 1 baud so a CW equivalent speed of 0.62 words/min.…and time delay of 30 seconds or more. It will be necessary for the ham to be patient!